Cation overcrowding effect on the oxygen evolution reaction

Jun Huang,1,2* Mengru Li,1 Mohammad J. Eslamibidgoli,2 Michael Eikerling,2,3 Axel Groß1,4

1 Institute of Theoretical Chemistry, Ulm University, 89069 Ulm, Germany

2 Institute of Energy and Climate Research, IEK-13: Theory and Computation of Energy Materials, Forschungszentrum Jülich GmbH, 52425 Jülich, Germany

3 Jülich Aachen Research Alliance: JARA-Energy, 52425 Jülich, Germany

4 Helmholtz Institute Ulm (HIU) Electrochemical Energy Storage, 89069 Ulm, Germany

Corresponding author, email: jun.huang@uni-ulm.de

# Abstract

The influence of electrolyte ions on the catalytic activity of electrode/electrolyte interfaces is a controversial topic for many electrocatalytic reactions. Herein, we focus on an effect that is usually neglected, namely, how the local reaction condition is shaped by non-specifically adsorbed cations. We scrutinize the oxygen evolution reaction (OER) at nickel (oxy)hydroxide catalysts, using a physico-chemical model that integrates density functional theory calculations, a microkinetic submodel, and a mean-field submodel of the electric double layer. The aptness of the model is verified by comparison with experiments. The robustness of model-based insights against uncertainties and variations in model parameters is examined, with a sensitivity analysis study using Monto-Carlo simulations. We interpret the decreasing OER activity with increasing effective size of electrolyte cations as a consequence of cation overcrowding near the negatively charged electrode surface. The same reasoning could explain why the OER activity increases with solution pH at the RHE scale, and why the OER activity decreases in the presence of bivalent cations. This work stresses the importance of correctly accounting for local reaction conditions in electrocatalytic reactions in order to obtain an accurate picture of factors that determine the electrode activity.

# Key words

Hydrogen production; Oxygen evolution reaction; Electrolyte cation effects; Local reaction condition; Surface charging effects; Electric double layer

# Introduction

Electrocatalysis is concerned, in the broadest sense, with the transformation between energy stored in chemical bonds and electrical energy. A hallmark of electrocatalysis is that the reaction rate, i.e., the rate of the transformation between chemical energy and electricity, can vary up to ten orders of magnitude on different electrode materials.1, 2 The electrode dependence of the reaction rate is largely ascribed to electronic interactions between the electrode and reaction intermediates.1-3 A quintessential descriptor of electronic interactions is the binding energy of reaction intermediates.1, 4 It is an important and vibrant direction of research that the reaction rate can be enhanced by tuning this descriptor, e.g., via harnessing strain and ligand effects.5, 6

Electrocatalytic reactions occur in a nanoscale interfacial region between the solid electrode and the electrolyte solution, termed electric double layer (EDL). Therefore, the reaction rate can also be tuned by modifying the composition of the electrolyte solution; relevant effects are termed electrolyte effects. Electrolyte effects have been observed in many electrocatalytic reactions, including hydrogen oxidation/evolution,7-10 oxygen evolution/reduction,7, 11-14 methanol electro-oxidation 15 and carbon dioxide reduction etc.16, 17 This work is concerned with the effects of electrolyte cations on the oxygen evolution reaction (OER), which is the performance-limiting anode reaction in electrochemical water splitting devices for hydrogen production.

The study of cation effects in the OER has a long history. Early studies were conducted by Erdey-Gruz and Shafarik in 1961 at platinum in 1M H2SO4 with 0.3-1 M of K+,Na+,Li+,Zn2+,Mg2+,Al3+,NH4+,18 and by Kozawa in 1964 at several metals in 1M NaOH with 0.1-10 mM of Ba2+,Cr2+,Ca2+.19 Erdey-Gruz and Shafarik observed that the activity decreases in the order K+<Al3+<NH4+<Zn2+<Na+<Mg2+<Li+.18 Kozawa found that the overpotential increases by 15~90 mV in presence of 0.1-10 mM of Ba2+,Cr2+,Ca2+.19 Subsequent interest in the cation effects on the OER came from the field of alkaline batteries around 1990s.20-23 More recent interests are sparked by the study of cation effects on hydrogen oxidation reaction and oxygen reduction reaction by Strmcnik et al.7

Cation effects on OER at NiOOH thin films in both purified and Fe-containing alkaline electrolytes are being actively studied, as Fe-doped NiOOH–based material is a promising catalyst for OER. In 2015, Michael et al. observed the activity trend of Cs+>K+>Na+≈Li+in purified electrolyte solutions, and K+≈Na+>Cs+>Li+ in Fe-containing electrolyte solutions.24 In 2017, Zaffran et al. observed that alkaline earth cations (Mg2+ and Ca2+) dramatically decrease the OER activity of at NiOOH thin films in both purified and Fe-containing alkaline electrolyte solutions, a modern echo of the results of Kozawa reported more than fifty years ago.25 Yang et al. further added tetraalkylammonium (TAA) cations into the comparison. They found an activity trend of K+≈Na+≈TBA+>TMA+>Li+ for the OER at NiOOH thin films in purified alkaline electrolyte solution, and a different active trend of K+≈Na+>TBA+>Li+>TMA+ in presence of Fe impurities.26 In 2019, Garcia et al. studied the effects of alkali metal cations on the OER at NiOOH thin films in both purified and Fe-containing alkaline electrolyte solutions. Their results show that the OER activity increases in the order of Cs+>Na+>K+>Li+ in purified 0.1M MOH (M=Cs, K, Na, Li), and Cs+>K+>Na+>Li+ in presence of Fe impurities.14 Perini and Ticianelli reported an activity order of K+>Na+>Li+ for the OER at gold.27 The same order of activity is found for the OER at a RuO2(110) single-crystal by Rao et al.28

It is certainly fair to say that the understanding of cation effects in the OER is still incomplete. Most studies have been focusing on the interaction of electrolyte cations with the OER intermediates. In 2015, Michael et al. measured Raman spectra during the linear scanning voltammetry in purified LiOH and CsOH. However, they pointed out that the “precise mechanism by which CsOH promoted better catalytic performance than LiOH is unclear with results from this study.”24 In 2017, Zaffran et al. assumed that the electrolyte cations are intercalated into the layered NiOOH structure. Their computational results showed that intercalated Mg2+ and Ca2+ bind strongly to water and OER intermediates, thus decreasing the OER activity.25 Yang et al. interpreted the cation effects from the perspective of interactions between cations and intermediates of the OER. They attributed the inhibition effect of TMA+ on the OER activity to the specific interaction of TMA+ with active oxygen-containing intermediates formed upon deprotonation.26 Later, Garcia et al. identified NiOO- as the active oxygen-containing intermediate using in situ surface enhanced Raman spectroscopy (SERS). Further, they showed that NiOO- is better stabilized by larger cations Cs+, which is believed to promote the OER.14 Contrarily, Rao et al. attributed the higher OER activity in the case of K+ to the weaker stabilization of active oxygen-containing intermediates, compared with Na+ and Li+, also using in situ SERS.28 In addition, they found that the density of isolated water molecules not hydrogen-bonded to other water molecule is highest, and the density of ice-like water molecules with four hydrogen bonds is lowest in the case of K+, compared with the cases of Na+ and Li+. More isolated water molecules promote the OER activity, they conjectured.

In this work, we are concerned with the effect of nonspecifically adsorbed cations, such as alkali metal cations,28 on the OER. The restriction to nonspecifically adsorbed cations allows us to focus on how electrolyte cations shape the local reaction condition in the EDL that differs dramatically from bulk conditions. The local reaction condition is deciphered using a model that integrates density-functional theory (DFT) calculations for elementary reaction steps, microkinetic modelling for multistep kinetics, and continuum modeling for the EDL structure. The model enables us to quantitatively correlate the OER activity and the effective cation size that considers the solvation shell. In addition, the model is extended to understand the effects of solution pH and bivalent alkaline earth cations on the OER activity. Given inevitable uncertainties and variations in model parameters, we conduct a sensitivity analysis study using Monte-Carlo simulations to gauge the robustness of the model-based findings. This work adds to the understanding of how electrocatalytic activity of the electrode is influenced by the local reaction condition.

# Methods

We adopt the concerted theory-computation framework, previously developed for oxygen reduction reaction by some of us,29 for the present case of the OER. The framework is composed of three parts. In the first part, we determine the reaction mechanism using quantum-mechanical DFT calculations. In the second part, we treat the multistep kinetics using microkinetic modelling. In the third part, we use a continuum EDL model to calculate the local reaction conditions. These parts are interrelated. The microkinetic model, on the one hand, is based on the reaction mechanism determined from DFT calculations, on the other hand, requires knowledge of the local reaction conditions that are determined from the EDL model. In turn, the EDL is influenced by interfacial reactions in that the surface charge density is a function of the coverages of reaction intermediates, which are the output of the microkinetic model. A consistent treatment of these models is a necessity for deciphering how electrolyte cations influence the OER, and is also a unique feature of this work in comparison with similar models for the OER 30-32 and other electrocatalytic reactions.33-35

## DFT calculations

Spin-polarized periodic first-principles calculations based on density functional theory (DFT) were performed with the projector-augmented-wave (PAW)36, 37 method using the Vienna ab initio simulation package (VASP)38-40. The Perdew-Burke-Ernzerhof (PBE) functional within the generalized gradient approximation (GGA) was used for exchange-correlation effects41. The wave functions were expanded by a plane wave basis with a kinetic cutoff energy of 750 eV. The DFT+U approach42, 43 was applied in order to correctly take account of the Coulomb interactions caused by the highly localized Ni 3d states of NiOOH, and the U-J value was set to be 5.5 eV, , which was calculated by Li and Selloni44, 45. Dispersion effects were taken into consideration using the DFT-D3 method46.

For the bulk optimization of NiOOH, a k-point mesh of 9 x 9 x 9 was used to integral over the first Brillouin zone. All atoms were relaxed to optimize until when the forces onto each atom were below 0.001 eV/Å. Here we used bulk β-NiOOH structure from our previous work44, and the lattice parameters were calculated to be a=2.92 Å, b= 5.84 Å, c= 4.82 Å (α=69.40°, β= 109.75°, γ=118.73°).

The β-NiOOH slab was modeled by the low-index (0001) termination in a three-layer slab with the bottom layer fixed, and was represented by a (2 x 4) supercell. The uppermost two layers were fully relaxed until convergence was reached. The convergence criterion was set to be 0.01 eV/Å for the maximal force onto each relaxed atoms. The magnetic moment for each Ni atom was initialized to be 1μB, and the resulting magnetic moments and related discussion are provided in section S4 in the Supporting Information (SI). A k-point mesh of 2 x 2 x 1 was applied in the slab calculation. Dipole corrections were considered to compensate for the interaction between surface dipoles and their periodic images. A vacuum layer of 25 Å was used to avoid any spurious interactions normal to the surface.

To describe the energetics of the OER on the β-NiOOH surface, the grand-canonical ab initio thermodynamics scheme47, 48 was used to evaluate the formation enthalpy of intermediates in solution. The computational hydrogen electrode (CHE) concept49, 50 was applied to calculate the electrochemical potential of H+ in solution, μH+, at a given electrode potential,

| μH++μe=12μH20-eUSHE-kBTln10pH=12μH20-eURHE, | (1) |
| --- | --- |

where μe is the electrochemical potential of the electrons, and μH20 that of hydrogen gas under standard conditions. URHE is the electrode potential on the reversible hydrogen electrode (RHE) scale. Using URHE instead of USHE versus the standard hydrogen electrode (SHE) is to align our results with the convention used in most of the experimental studies. The electrochemical potential of OH- can be derived from,

| μOH-=μH2O-μH+. | (2) |
| --- | --- |

First, we calculated the Gibbs free energy change for each elementary step of OER, and explored the most favorable pathway under standard conditions (T=298K, pH = 0, pH2=1 bar, 0 VSHE). Multiple experimental studies have revealed that the original surface undergoes surface deprotonation prior to the OER.14, 51, 52 Accordingly, the first step of the OER is thus,

| O-Ni-OH + OH-↔O-Ni-O + H2O + e, | (3) |
| --- | --- |

where O-Ni-OH represents the original surface, and O-Ni-O is the (2 x2) surface with ¼ hydrogen atoms deprotonated.

The second step is the adsorption of a hydroxyl anion (OH-) onto the deprotonated site, forming O-Ni-OOH,

| O-Ni-O + OH-↔O-Ni-OOH + e. | (4) |
| --- | --- |

Afterwards, another OH- comes and removes one H from O-Ni-OOH,

| O-Ni-OOH + OH-↔O-Ni-OO + H2O + e, | (5) |
| --- | --- |

followed by a chemical step releasing O2, leaving the surface O-Ni-⊡ with an oxygen vacancy,

| O-Ni-OO↔O-Ni-⊡ + O2. | (6) |
| --- | --- |

Finally, the initial surface is recovered by filling the oxygen vacancy site with a OH- in solution, completing the reaction cycle,

| O-Ni-⊡ + OH-↔O-Ni-OH + e. | (7) |
| --- | --- |

The Gibbs free energy changes of the elementary steps as a function of the electrochemical control parameters were derived using the CHE method.49, 50 Steps (3)(4)(5)(7) proceed via OH- coupled electron transfer steps, for which we need the electrochemical potential of the electron-OH- pair,

| μOH--μe=μH2O0-12μH20+eURHE | (8) |
| --- | --- |

where μOH- is the electrochemical potential of hydroxyl anions, μH2O0 that of liquid water under standard state. In order to compute μH2O0 and μH20, we calculated the total energy of a single water molecule and a single hydrogen molecule using DFT, and corrected for entropic contributions using entropy data from NIST Chemistry Webbook.53

Consider, for example, the first deprotonation step. The Gibbs free energy change of this step at any potential URHE is calculated as,

| ΔG1=EO-Ni-O+μH2O0-EO-Ni-OH-μOH--μe =EO-Ni-O-EO-Ni-OH+12μH20-eURHE, | (9) |
| --- | --- |

where we used Eq.(8) for μOH--μe, EO-Ni-O and EO-Ni-OH represent the total energies of the O-Ni-O and O-Ni-OH surfaces calculated using quantum-mechanical DFT. The Gibbs free energy change of other steps can be calculated in the same manner, expressed as,

| ΔG2=EO-Ni-OOH-EO-Ni-OH-μH2O0+12μH20-eURHE, | (10) |
| --- | --- |
| ΔG3=EO-Ni-OO-EO-Ni-OOH+12μH20-eURHE, | (11) |
| ΔG4=EO-Ni-⊡-EO-Ni-OO+4.92+2μH2O0-μH20, | (12) |
| ΔG5=EO-Ni-OH-EO-Ni-⊡-μH2O0+12μH20-eURHE, | (13) |

where we used μO20=4.92+2μH2O0-μH20, in Equation (12).

The Gibbs free energy profile of the OER calculated under standard conditions (298K, pH = 0, 1 bar, 0 VSHE) is shown in Figure 1. The first deprotonation step has the most positive Gibbs free energy change, ΔG10=1.91 eV, which means that it corresponds to the potential determining step, which is consistent with the results of Govind-Rajan et al.54 The following four steps are energetically feasible at the equilibrium potential of OER, E=1.23 VRHE. Especially, the NiO state with an oxygen vacancy is highly unstable and inclined to adsorb a OH- from solution. Therefore, we combine the step 4 and 5 into a single step in the microkinetic model.

Figure 1. Gibbs Free energy profile of OER on  β-NiOOH (0001) surface calculated at the standard state (298K, pH = 0, 1 bar, 0 VSHE).

## Microkinetic model

A widely assumed reaction pathway of the OER in alkaline media proceeds via four OH--coupled electron transfer steps,55-57

| OH+OH-↔O+H2O+e, | (14) |
| --- | --- |
| O+OH-↔OOH+e, | (15) |
| OOH+OH-↔OO+H2O+e, | (16) |
| OO+OH-↔OH+O2+e. | (17) |

The reaction mechanism expressed in Equations (3)-(7) can be considered a specific case of the general mechanism, with Equation (14) corresponding to Equation (3), Equation (15) to Equation (4), Equation (16) to Equation (5), Equation (17) to the combination of Equations (6) and (7) because the NiO state is highly energetically unstable.

For the sake of generality, the microkinetic model is formulated for the general mechanism expressed in Equations (14)-(17). The reaction rates are given by,

| v1=k1+OH-'θOH-k1-θO, | (18) |
| --- | --- |
| v2=k2+OH-'θO-k2-θOOH, | (19) |
| v3=k3+OH-'θOOH-k3-θOO, | (20) |
| v4=k4+OH-'θOO-k4-θOH. | (21) |

Here, OH-' represents the concentration of OH- at the reaction plane, which is to be determined from the EDL model; ki± represent the forward (+) and backward (-) rate constants of ith step, which are dependent on the local reaction condition provided by the EDL model; θOH, θO, θOOH, θOO represent the coverages of reaction intermediates, respectively, constrained by the conservation of surface sites,

| θOH+θO+θOOH+θOO=1. | (22) |
| --- | --- |

According to the transition state theory, the rate constants are given by,

| ki±=kBThexp-Ga,i±kBT, | (23) |
| --- | --- |

where the activation barriers are related to the Gibbs free energies of reactions according to the Bronsted-Evans-Polanyi (BEP) relation,

| Ga,i+=Ga,i⊝+βi∆∆Gi, Ga,i-=Ga,i⊝-1-βi∆∆Gi, | (24) |
| --- | --- |

where Ga,i⊝ is the activation barrier under standard equilibrium conditions (298K, pH = 0, 1 bar), denoted by a superscript ‘⊝’. ∆∆Gi represents the variation in the Gibbs free energies of reactions when the reaction condition deviates from the standard equilibrium conditions. We shall also specify the coverages of adsorbates under the standard equilibrium conditions, if we attempt to consider the coverage-dependence of ∆∆Gi. In our DFT calculations, we have θO⊝=θOO⊝=θOOH⊝=1/4 because one of the four sites in the supercell is subject to surface reactions, and θOH⊝=1 for the original surface.

Under any reaction condition, Δ∆Gi is given by,

| Δ∆G1=-e0ϕM-ϕ'-U1⊝+ΔΔGO-ΔΔGOH, | (25) |
| --- | --- |
| Δ∆G2=-e0ϕM-ϕ'-U2⊝+ΔΔGOOH-ΔΔGO, | (26) |
| ∆ΔG3=-e0ϕM-ϕ'-U3⊝+ΔΔGOO-ΔΔGOOH, | (27) |
| ∆ΔG4=-e0ϕM-ϕ'-U4⊝+ΔΔGOH-ΔΔGOO, | (28) |

where ϕM represents the electrode potential on the SHE scale, and ϕ' is the potential at the reaction plane in solution relative to the electric potential in the solution bulk which is taken as the potential reference.

Ui⊝’s, the standard equilibrium potentials of the reaction steps in Equations (14) - (17) versus the RHE, are given by,

| U1⊝=1e0EO-Ni-O-EO-Ni-OH+12μH20, | (29) |
| --- | --- |
| U2⊝=1e0EO-Ni-OOH-EO-Ni-OH-μH2O0+12μH20, | (30) |
| U3⊝=1e0EO-Ni-OO-EO-Ni-OOH+12μH20, | (31) |
| U4⊝=1e0EO-Ni-OH-EO-Ni-OO+4.92+μH2O0-12μH20, | (32) |

based on Equations (9)-(13).

ΔΔGi in Equations (25)-(28) accounts for the variation in Gibbs free energies of the reaction intermediates OH, O, OOH, and OO when ϕM changes. There are multiple reasons for such variations. On the one hand, θOH, θO, θOOH, θOO changes with ϕM, so the Gibbs free energy of adding a new intermediate changes due to lateral interactions of the new intermediate with existing ones.58 On the other hand, the local electric potential and electric field change with ϕM, altering the Gibbs free energy of intermediates that carry a net charge. The Frumkin adsorption isotherm uses an empirical coefficient, called the lateral interaction coefficient, to describe the linear relation between ΔΔGX and θX. The linear relation is a first approximation. In addition, the Frumkin adsorption isotherm usually neglects interactions among adsorbed intermediates of different kinds. Norskov and coworkers expressed ΔΔGX as a function of the electric field, whose coefficients are fitted from DFT calculations.59, 60 In an empirical sense, the Frumkin adsorption isotherm and the relation of Norskov and coworkers are equivalent, as there exists a one to one correspondence between θX and the electric field, both of which are uniquely determined by ϕM. Herein, we use,

| ∆ΔGX=μXEAP, | (33) |
| --- | --- |

with μX an empirical dipole moment of intermediate X, and EAP being the electric field strength at the adsorbate plane (AP). As will be made clear in the EDL model, EIHP is co-determined by all existing adsorbed intermediates. Therefore, equation (33) effectively considers interactions among adsorbed intermediates of all kinds.

At steady state, we have,

| v1=v2=v3=v4=jOER4e0ρs, | (34) |
| --- | --- |

where jOER is the current density of the OER, and ρs the number density of active sites.

We can solve for the coverages,

| θOH=1Ξ1k4-+K4k3-+K3K4k2-+K2K3K4k1-, θO=1Ξ1k1-+K1k4-+K1K4k3-+K1K4K3k2-, θOOH=1Ξ1k2-+K2k1-+K2K1k4-+K2K1K4k3-, θOO=1Ξ1k3-+K3k2-+K3K2k1-+K3K2K1k4-, | (35) |
| --- | --- |

with Ki=ki+*/ki-, ki+*=ki+OH-', and Ξ,

| Ξ=1+K1+K1K2+K1K2K3k4-+1+K4+K4K1+K4K1K2k3- +1+K3+K3K4+K3K4K1k2-+1+K2+K2K3+K2K3K4k1-. | (36) |
| --- | --- |

The current density is transformed to,

| 4eρSjOER=Θ1k1+,*+Θ2k2+,*+Θ3k3+,*+Θ4k4+,*, | (37) |
| --- | --- |

with the thermodynamic factors

| Θ1=1+K2+K2K3+K2K3K4K2K3K4, Θ2=1+K3+K3K4+K3K4K1K1K3K4, Θ3=1+K4+K4K1+K4K1K2K1K2K4, Θ4=1+K1+K1K2+K1K2K3K1K2K3. | (38) |
| --- | --- |

## Electric double layer model

We require an EDL model to compute the local reaction condition, including ϕ', OH-', and EAP. The mean-field Helmholtz free energy per unit volume of the electrolyte solution is written as,61, 62

| fes=e0ϕ(Nc-Na)-ϵ∞E22-1βNslnsinhpEβpEβ+1βlnN!Nc!Na!Ns! | (39) |
| --- | --- |

where e0 is the elementary charge, ϵ∞ the optical component of the dielectric constant, ϕ the potential, E=∇ϕ the electrical field, β=1/kBT the inverse thermal energy, p the dipole moment of solvent molecules, N the total number density of lattice sites, Ni (i=s,a,c) the number densities of solvent molecules (s), anions (a), and cations (c). On the right-hand side of Eq.(39), the first term is the electrostatic free energy of ions, the second term the self-energy of the electric field, the third term the electrostatic free energy of solvent molecules, and the last term the entropic free energy related to the configuration of solution species, which is calculated using a lattice-gas approach. The conservation of lattice sites leads to Ns=N-Na-Nc.

Based on the variational principle, we obtain the following Euler-Lagrange equation in terms of ϕ,

| ∂fes∂ϕ-∂∂x∂fes∂E=0, | (40) |
| --- | --- |

leading to a modified Poisson-Boltzmann (PB) equation,

| -∂∂xϵ∞+pNsELpEkBTE=e0Nc-Na, | (41) |
| --- | --- |

where Lx=coth(x)-1x is the Langevin function. Compared with the conventional PB equation, Eq.(41) features an electric field-dependent local dielectric constant. Equation (41) has been derived in the dipolar Poisson-Boltzmann model by Abrashkin et al.,63 and the Langvein-Poisson-Boltzmann model by Gongadze and Iglic.64

The distributions of cations and anions are obtained from

| ∂fes∂Nc=∂fes∂Ncbulk, | (42) |
| --- | --- |
| ∂fes∂Na=∂fes∂Nabulk, | (43) |

namely, electrochemical potentials of cations and anions in the EDL are uniform and thus equal to their bulk values. Substituting Eq. (39) into Eq. (42) and (43) leads to,

| Nc=Nχexp-βe0ϕχγcexp-βe0ϕ+χγaexpβe0ϕ+(1-2χ)sinhpEβpEβ, | (44) |
| --- | --- |
| Na=Nχexpβe0ϕχγcexp-βe0ϕ+χγaexpβe0ϕ+(1-2χ)sinhpEβpEβ, | (45) |

where χ is the bulk number density of cations/anions normalized to N, γa and γc denote relative sizes of solvated anions and cations, respectively, calculated as

| γa,c=da,cds3, | (46) |
| --- | --- |

with da,c being the effective diameters of solvated anions and cations, and ds the diameter of solvent molecules, which is taken as the reference size of the lattice. We stress that equations (44) and (45) take into account the ion size effect in a phenomenological manner, namely, by introducing two size factors γa and γc. A more rigorous treatment of the size effect can be found in ref.65.

Substituting Eq.(44) and Eq. (45) into Eq.(41) leads to a single differential equation governing the distribution of electric potential. Its boundary conditions are as follows,

| ϕ=0, | (47) |
| --- | --- |

in the solution bulk, and

| ϕ=ϕM-ϕpzc+δAPEAP, | (48) |
| --- | --- |

at the adsorbate plane (AP), which is designated as the coordinate origin, x=0, of the EDL model. The adsorbate layer has a thickness of δAP, a permittivity of ϵAP, and an electric field of EAP. ϕpzc is the potential of zero charge. The third term on the right hand side (RHS) of Eq.(48) represents the potential drop across the adsorbate layer, where the uniform electric field EAP is given by,66, 67

| EAP=ϵϵAP∂ϕ∂x+μchemϵAPδAP, | (49) |
| --- | --- |

where the first term on the RHS represents the electric field caused by the excess free charge on the electrode surface, and the second term represents the electric field caused by the dipole moment formed between charged intermediates and its compensating charge in the electrode, denoted by μchem,66, 67

| μchem=X=OH, O,OOH,OOθXξXδAPρs | (50) |
| --- | --- |

where ξX is the net charge number of adsorbed intermediates.

## Model parameters and numerical implementation

Table 1 lists the model parameters which are categorized into five subgroups termed “general constants”, “electrode”, “electrolyte solution”, “interfacial structure”, and “reaction”. The electrode parameters correspond to β-NiOOH (0001). The (2 x 2) supercell has a surface area of Ssc=(5.85 Å x 10.24 Å) = 59.90 Å2, according to the DFT calculation. Therefore, the number density of reaction sites ρs=4Ssc=6.67×1018 m-2. The work function of the slab surface with one layer of water molecules is calculated to be Φ=5.6 eV.44 Therefore, the pzc versus the SHE is calculated as ϕpzc=Φe0-4.44=1.16 VSHE, assuming that the absolute electrode potential of the SHE is 4.44 eV.

The electrolyte parameters correspond to 0.1 M MOH (M=Li, Na, Cs). From the expression of the effective dielectric constant given in equation (41), we obtain the bulk dielectric constant as ϵb=ϵ∞+13p2NskBT. Therefore, the effective dipole moment of water molecules is found as 3kBTϵb-ϵ∞csbNA. The effective size of solvated ions can be controversial, due to the complicated solvation structure of ions and the uncertainty in defining the effective size. Herein, we cite Israelachvili’s list of hydrated radii of ions in water, da=3.0 Å for hydrated OH-, and dc=7.6,  7.2,  6.6,  6.6 Å for hydrated Li+, Na+, K+, Cs+, respectively.68 The adopted values are close to the Stokes radii of hydrated ions in water from ionic conductivity measurements.10

As a first approximation, we assume that all adsorbates line up rigidly in the adsorbate layer and are equidistant from the electrode surface. The thickness of the adsorbate layer is δAP=1.5 Å as the mean value of the sizes of OH, O, OOH, and OO. The permittivity of the adsorbate layer is usually assumed to be several folds of the vacuum permittivity, and here we inherit ϵAP=3ϵ0 from ref.67.

The standard equilibrium potentials of the reaction steps versus the RHE are calculated from equations (29)-(32) with EO-Ni-O, EO-Ni-OOH, EO-Ni-OO, EO-Ni-⊡, μH2O0, and μH20 obtained from DFT calculations. The Kohn-Sham DFT approach implemented with PBE and RPBE exchange-correlation functionals has a typical root-mean-square deviation of ~0.3 eV for chemisorbed systems.69 The error shall grow in electrochemical systems with the presence of the electrolyte solution. In addition, we have neglected the zero-point energy corrections, see discussion in section S3 in the SI. Therefore, the values of Ui⊝ calculated with DFT results are allowed to vary in the range of ~0.3 V. The transfer coefficients of the reaction steps are taken as βi=0.5. Microscopically, βi shall vary as a function of ϕM and vary from reaction to reaction.70, 71 In Ref. 70, Huang derived an analytical expression for βi where different factors are discriminated. However, since we use the phenomenological Butler-Volmer equations for the elementary step kinetics, we should not be bothered by mechanistic complexities of βi. The nudged elastic band (NEB) method can be used to calculate activation barriers of electrocatalytic reactions.72 Herein, we choose to fit the activation barriers from experimental polarization data, considering that the error in DFT calculations at the transition state should be larger than that at the ground state, namely, >0.3 eV. The net charge numbers of adsorbed intermediates ξX are also fitted from experimental data. The fitting details are provided in section S1 of the SI. The sensitivity of the fitted parameters will be gauged at a latter point in the discussion section. The numerical implementation of the model was performed in MATLAB. The code script is available upon request. We note that all potentials are rescaled to the RHE in the model calculation.

Table 1 Model parameters for the base case

| General constants |  |  |
| --- | --- | --- |
| kB, Boltzmann constant | 1.38∙10-23 J/K |  |
| e0, elementary charge | 1.6×10-19C |  |
| NA, Avogadro constant | 6.02×1023/mol |  |
| ϵ0, vacuum permittivity | 8.85×10-11F/m |  |
| ϵb, permittivity of bulk water | 78.5ϵ0 |  |
| h, Planck constant | 6.626×10-34J/s |  |
| T, temperature | 298.15 K |  |
| Electrode |  |  |
| ρs, the number density of reaction sites | 6.67×1018 /m2 | DFT calculation |
| Φ, work function of the solvated electrode | 5.6 eV | DFT calculation44 |
| ϕpzcvs RHE, potential of zero charge | Φe0-4.44+ln10kBTe0pH | Absolute electrode potential of the SHE is 4.44 eV |
| Electrolyte solution |  |  |
| ds, diameter of water molecules | 2.75 Å |  |
| da, effective diameter of solvated anions | 6.0 Å |  |
| dc, effective diameter of solvated cations | 7.6, 7.2, 6.6, 6.6 Å for Li+, Na+, K+, Cs+ |  |
| ϵ∞, optical permittivity | ϵ0 |  |
| p, effective dipole moment of water molecules | 3kBTϵb-ϵ∞csbNA |  |
| csb, bulk concentration of water | 55.5 M |  |
| pH | 13 |  |
| ca/cb, bulk concentration of anions/cations | 0.1 M |  |
| N, total number density of lattice sites | csb+cab+ccbNA |  |
| χ, bulk number density of cations/anions normalized to N | cabNAN |  |
| Interfacial structure |  |  |
| ϵAP, permittivity of the adsorbate layer | 4ϵ0 |  |
| δAP, thickness of the adsorbate layer | 1.5 Å |  |
| Reaction |  |  |
| Ui⊝, standard equilibrium potentials of the reaction steps versus RHE | 1.92, 1.22, 1.18, 0.60 V for Equations (14) - (17), respectively | DFT calculation |
| βi, transfer coefficient | 0.5 |  |
| Ga,i⊝, activation barrier under the standard equilibrium conditions | 0.50, 0.33, 0.31, 0.44 eV for Equations (14) - (17), respectively | Fitted |
| μX, empirical dipole moment of intermediate X | ξXe0δAP | Fitted |
| ξX, net charge number of adsorbed intermediates | -0.06, -0.01,-0.04,-0.04 for OH, O, OOH, OO, respectively | Fitted |

# Results

In this section, we describe how the local reaction condition is shaped by the cations in the first subsection, which lays the basis for understanding the cation effects on the OER in the second subsection. Afterwards, we discuss other means to tune the cation effects, including the solution pH in the third subsection, and bivalent cations in the last subsection.

## Local reaction conditions

The local reaction condition computed by the model is shown in Figure 2, where the effective diameter of the cations dc is varied between 4 Å and 8 Å with a step of 0.4 Å and eleven lines are included in each figure. Figure 2 (a)-(c) exhibits the spatial distributions of OH- concentration (OH-), cation concentration(C+), and dielectric constant (ϵ) in the EDL at ϕM=1.6 VRHE, respectively. Let us first analyze general trends and then dissect differences among different lines corresponding to different dc’s.

OH- decreases from the solution bulk to the electrode surface because the surface charge density σ is negative even at this high electrode potentials, see Figure 2 (d), and OH- ions are repelled as co-ions away from the electrode surface. σ is negative in the considered potential range because the pzc is 1.93 VRHE for pH=13, which is consistent with the fact that the point of zero charge of Ni(OH)2 is ~11 (namely, the solid will be negatively charged at pH>11) .73 Consequently, cations are attracted as counterions by the negative surface charge, resulting in an increasing profile of C+ toward the electrode surface. In addition, ϵ decreases towards the electrode surface, which is ascribed to the combined effect of polarization saturation due to the high interfacial electric field, and density decrement due to the crowding of counterions. The monotonically decreasing trend of ϵ in the EDL has been revealed by Nakayama and Andelman,74 and Gongadze and Iglič.64 In a previous work, Huang, Chen and Eikerling showed nonmonotonic profiles of ϵ in the EDL due to the competition between ions and solvent molecules.62 The nonmonotonic behavior is more pronounced in the presence of specific ion adsorption.

As dc increases, less cations can accumulate in the EDL and C+ decreases accordingly, as shown in Figure 2 (b). Moreover, the black line corresponding to dc=8 Å levels off near the electrode surface, signifying the overcrowding phenomenon of cations.75 Due to the intensified steric repulsion of cations, both OH- and ϵ decrease for larger dc, see Figure 2 (a) and (c), respectively. Hereinafter, the cation overcrowding effects are referred to as effects caused by the substantial accumulation of cations near the electrode surface due to the negative surface charge, including decreased OH-, lower ϵ and among others.

Figure 2 (d)-(f) exhibits the variation of surface charge density (σ), OH- concentration at x=0 (OH-'), and electric potential at x=0 (ϕ') as a function of ϕM, respectively. σ increases toward positive values and OH-' grows with increasing ϕM. The larger the cation size, the smaller the magnitude of σ. The dependence of σ on the cation size is readily understood from the effective thickness of the diffuse layer, which becomes thicker for larger cations, resulting in a smaller magnitude of σ. A similar analysis is provided by Ringe et al. for cation effects in carbon dioxide reduction.76 For the case of a larger dc, OH-' decreases due to greater steric repulsion because the size factor γc is cubic with dc, in spite of the decreasing magnitude of σ. Figure 2 (e) shows that the local electric potential ϕ' follows the positive-shift trend of σ with increasing ϕM, and grows in the magnitude with increasing dc due to diminished screening effect of water molecules, Figure 2 (c).

Figure 2. Local reaction conditions of the OER with the effective cation diameter dc varying between 4 Å and 8 Å with a step of 0.4 Å. (a)-(c) Distribution of OH- concentration ([OH-]), cation concentration([C+]), and dielectric constant (ϵ) in the EDL at ϕM=1.6 VRHE, respectively. The coordinate origin x=0 corresponds to the outer plane of the adlayer. (d)-(f) Variation of surface charge density (σ), OH- concentration at x=0 (OH-'), and electric potential at x=0 (ϕ') as a function of ϕM, respectively. Model parameters are listed in Table 1.

## Model-experiment comparison

With a knowledge of cation effects on the local reaction conditions, we proceed to analyze cation effects on the polarization curve, namely, the voltage-current relation. Figure 3 (a) shows that the exponential growth in jOER with increasing ϕM is deferred for Li+ compared with Cs+, which is consistent with experimental data, Figure 3 (b).14, 24 However, we also notice deviations in the details of the polarization curves between the model and experiments. Possible causes for the deviations are many, including the deviation between the idealized β-NiOOH(0001) slab and the real surface states of the oxide, the deviation between the phenomenological Butler-Volmer equation and the complicated electron transfer kinetics, and the neglect of mass transport effects in this model. In Figure S2, we compare the polarization curves of OER at NiOOH in purified 0.1 MOH (M=Li+, Na+, Cs+) measured by two laboratories.14, 24 Though the activity trend of Cs+>Na+>Li+ is shared by both, noticeable deviations in the magnitude of the current density and the detailed profiles of the polarization curves are found. Nevertheless, the deviations in details of the polarization curves between the model and experiments in Figure 3 and between two experimental studies in Figure S2 shall not prevent us from understanding the trend of cation effects on the OER.

The microkinetic model enables us to gain deep insights into the reaction mechanism. Figure 3 (c) shows the variation of different resistance terms Θiki+,* (i=1,2,3,4) in Equation (37) as a function of ϕM. The rate-determining term (RDT) transitions from the third term i=3 to the second term and then to the first term as ϕM increases. The RDT above 1.6 VSHE, namely, the first term, grows in the order Cs+<Na+<Li+, Figure 3 (d), leading to the catalytic activity decreasing in the order Cs+>Na+>Li+, Figure 3 (a). The reason is simply that larger cations occupy more volume near the negatively charged surface, leading to a smaller concentration of OH-, the reactant, Figure 2 (a). In Figure S1, we analyze an extreme case where the activation energy of the first step is increased to 0.98 eV, around 0.6 eV higher than other steps, in order to provide the readers an example of how this model behaves in extreme cases.

Figure 3. Model-experiment comparison. (a) Model-calculated polarization curves for OER in 0.1 MOH (M=Li+, Na+, Cs+) with model parameters listed in Table 1. (b) Experimental polarization curves under the same condition measured by Michael et al.24 (c) Four resistance terms Θiki+,* in Equation (37) as a function of the electrode potential for the case of M=Li+. (d) The rate-determining resistance above 1.6 VRHE, Θ1k1+,*, for different cations.

## pH effects

For proton-coupled electron transfer (PCET) reactions, under thermodynamic equilibrium conditions the reaction should be invariant on the RHE scale.77, 78 Therefore, any observed pH effects on the RHE scale are usually ascribed to decoupled proton and electron transfer reactions.79 Note that our current model goes beyond such a purely thermodynamic analysis. We will show below that in fact microkinetic considerations influenced by double-layer effects bring new insights into the pH effects in the OER.

Figure 4 (a) shows that the OER activity on the RHE scale increases with increasing pH, which is consistent with the experimental data.79, 80 The total ionic strength is 1 M for all pH values considered. The pH-dependent trend in activity is in contrast with a thermodynamic analysis that predicts a pH-independent activity on the RHE scale as all elementary steps are PCET reactions in our model.77 The discrepancy lies in the EDL effects. Figure 4 (b) shows that σ is more negative at higher pHs, which can be effectively understood by the fact that the pzc on the RHE scale increases with pH. Therefore, the cation overcrowding effect is more pronounced at higher pHs, resulting in a lower OH-' at potentials below 1.6 VRHE, even though the bulk OH- concentration is greater at higher pHs. σ is less negative with increasing ϕM, alleviating the cation overcrowding effect and leading to larger OH-' for higher pHs at potentials above 1.6 VRHE. Since the OER activity is limited by OH-' that is dictated by the cation overcrowding effect, the OER activity thus grows with increasing pH at potentials above 1.6 VRHE, Figure 4 (a).

Figure 4. (a) pH effect of the OER in x (x=1, 0.1, 0.01) M LiOH with the total ionic strength kept at 1 M. (b) The surface charging relation for the three pHs. (c) OH- concentration at the reaction plane as a function of the electrode potential for the three pHs.

## Bivalent cation effects

Adding bivalent cations into the solution while retaining the same OH- concentration, we observe a decrement in the OER activity, as shown in Figure 5 (a), which is consistent with the experimental observations.19, 25 The underlying causes are related to the modifications to the EDL by the bivalent cations. Figure 5 (b) exhibits the surface charging curves. Specifically, σ is slightly more negative above 1.6 VRHE in the presence of Ca2+. Due to the larger size of Ca2+ (8.2 Å in diameter), OH-' is lower in the presence of Ca2+, Figure 5 (c), leading to the lower OER activity in Figure 5 (a). Figure 5 (d) and (e) show the spatial distributions of OH- concentration and cation concentrations in the EDL at 1.6 VRHE. As bivalent cations are more effective in screening the electric field, the concentration of Ca2+ is greater near the electrode surface than that of Cs+. Consequently, the OH- concentration near the electrode surface is lower in the presence of Ca2+, Figure 5 (d).

Figure 5. (a) Influence of bivalent cations (Ca2+) on the OER. Two electrolyte solutions are compared, including 0.1 M CsOH (blue) of 0.08 M CsOH + 0.01 M Ca(OH)2 (red). The OH- concentration is kept at 0.1 M for both electrolyte solutions. (b) Comparison of the surface charging relation. (c) Comparison of the OH- concentration at the reaction plane as a function of the electrode potential. (d) Spatial distribution of OH- concentration at 1.6 VRHE. (e) Spatial distribution of monovalent (solid lines) and bivalent cations (dashed lines) at 1.6 VRHE.

# Discussion

In this section, we first compare electronic and electrostatic factors regarding the cation effects. Then, we gauge the robustness of the model results in the vast parameter space by examining the sensitivity of the model output to the variation/uncertainty of the model parameters. Finally, we dissect features and limitations of the model and discuss the extensions needed when it is applied to other catalysts and reactions.

## Electronic vs. electrostatic factors

The model considers both electronic and electrostatic factors. The former type is reflected in the binding energies under standard conditions and dipole corrections, namely, the ΔΔGi terms. The latter type includes local electric potential and concentrations that are influenced by electrostatic interactions. The key property governing the electrostatic interactions is the surface charge density, which is modified by chemisorption-induced surface dipoles (μchem). In this section, we compare the importance of electronic and electrostatic factors for the case of the cation effect in the OER.

In Figure 6 (a), we compare three cases, including the base case, the second case without dipole corrections for the binding energies (ΔΔGi=0), and the third case without chemisorption-induced surface dipoles that is realized by setting χi=0. The comparison is made in terms of the OER current density at 1.7 VSHE for a series of dc referenced to that for dc=8 Å. All curves show that the OER current density decreases as dc increases. The curves for the base case and the second case of ΔΔGi=0 are indistinguishable, while the curve for the third case of χi=0 is markedly suppressed. This indicates that ΔΔGi is unimportant here and the cation size effect is mainly ascribed to the surface charging effects. The surface charging relation for the third case of χi=0 is shown in Figure 6 (b). Compared with Figure 2 (d) for the base case, Figure 6 (b) exhibits a reduced difference of σ for different dc’s, thus leading to a much reduced dc-dependence of the OER current in Figure 6 (a). In addition, compared with the base case, σ has a smaller magnitude for χi=0, because negatively charged adsorbates increase the pzc of the electrode.66 As a result, the cation overcrowding effect is diminished for the third case of χi=0.

Our argument that the cation effect on the OER is mainly caused by surface charging effects instead of the shift in the field-dependent binding energies is further supported by the results presented in Figure 6 (c). The x-axis is the potential of zero charge which dictates the surface charging relation. The y-axis is the serial number of Monte-Carlo simulations in which U1⊝,U2⊝,U3⊝ are allowed to vary randomly within 0.2 V around their base values. U4 is given by the constraint condition that U4⊝=4.92-U1⊝-U2⊝-U4⊝. It is clearly shown that the OER current ratio between Cs+ and Li+ at 1.7 VSHE, j1.7 VRHECs+j1.7 VRHELi+, increases significantly when the pzc grows, namely, when the electrode surface gets more negatively charged. On the contrary, j1.7 VRHECs+j1.7 VRHELi+ is slightly affected by random variations in the equilibrium potentials of the elementary steps which are determined by the binding energies of the adsorbates. The insensitivity of the cation effects to binding energies is in accord with the robustness of the cation effects observed in a wide range of catalysts in experiments, including Ni(Fe)OOH, RuO2, Au, IrO2, and perovskites.14, 24-26, 28

Continuing in our line of reasoning, we deduce that the effect of cation size will be less significant for catalysts with a lower pzc. Titanium dioxide (TiO2) has a work function of 5.4 eV, around 1 eV lower than that of nickel oxides. Based on the linear relation between the work function and the pzc, TiO2 shall carry positive charge on its surface during the OER. Therefore, if electrostatic factors, more precisely, the surface-charge modulated cation overcrowding effects, dominate, we expect that the cation size effect on the OER is much weaker at TiO2 compared with NiO. A systematic comparison of the cation effects on different catalysts with different work functions is important to examine the importance of surface charging effects in the cation effects on the OER.

Figure 6. (a) Comparison between the base case, the case without dipole corrections for the binding energies (ΔΔGi=0), and the case without chemisorption-induced surface dipoles (χi=0). The vertical axis represents the OER current density at 1.7 VRHE for a certain value of dc referenced to that for dc=8 Å. (b) Surface charging relation for the case without chemisorption-induced surface dipoles. (c) The OER current ratio between Cs+ and Li+ at 1.7 VRHE. The x-axis is the potential of zero charge. The y-axis is the serial number of simulations in which U1⊝,U2⊝,U3⊝ are allowed to vary randomly within 0.2 V around their base values, with U4⊝=4.92-U1⊝-U2⊝-U4⊝.

## Parameter sensitivity and robustness of the model results

Given the significant number of parameters in our model, we shall dissect the robustness of the model results against uncertainties and variations in model parameters. To this end, a parameter sensitivity study is conducted. Among model parameters in Table 1, those having significant uncertainties and variations include the pzc, the equilibrium potentials Ui⊝ (equivalently, the binding energies), the activation barriers Ga,i⊝, the transfer coefficients (βi), and the net charge numbers of adsorbed intermediates ξi. In the preceding subsection, we have shown that the cation effects are highly sensitive to the pzc, and robust against variations in Ui⊝. In this subsection, we complement the parametric analysis by examining the influence of Ga,i⊝, ξi and βi. The output of the model used for the sensitivity analysis is the current density ratio between Cs+ and Li+ at 1.7 VRHE, denoted as j1.7 VRHECs+j1.7 VRHELi+. Three groups of Monte-Carlo simulations are conducted. Each group consists of 100 runs of the model. In each run, the four Ga,i⊝’s are varied randomly in the range of [0.3, 0.6] eV in the first group, the four ξi’s are varied randomly in the range of [-0.1, 0] in the second group, and βi is varied randomly in the range of [0.3, 0.7] in the third group.

The histogram of the Monte-Carlo simulation is exhibited in Figure 7. All data points of j1.7 VRHECs+j1.7 VRHELi+ in the first group are above 2, indicating that the trend of higher OER activity at smaller dc is robust against variations in Ga,i⊝’s. The data points in the second group are more dispersed, indicating that j1.7 VRHECs+j1.7 VRHELi+ is highly sensitive to the four ξi’s because they determine the surface charging relation and thus the cation overcrowding phenomenon, as has been analyzed for the third case in Figure 6 (a). This further evidences our claim made in the preceding subsection that alkali metal cations influence the OER activity mainly through modulating the local reaction conditions that are determined by the surface charge density. The distribution of data points in the third group is much smaller, indicating that j1.7 VRHECs+j1.7 VRHELi+ is less sensitive to the transfer coefficients.

Figure 7. Histogram of the Monte-Carlo simulation. The horizontal axis is current density ratio between Cs+ and Li+ at 1.7 VRHE. Three groups of Monte-Carlo simulation are conducted. Each group consists of 100 runs of the model. In each run, the four Ga,i⊝’s are varied randomly in the range of [0.3, 0.6] eV in the first group, the four ξi’s are varied randomly in the range of [-0.1, 0] in the second group, and the transfer coefficient βi is varied randomly in the range of [0.3,0.7]. Other parameters have their base values as listed in Table 1.

## Features, limitations and extensions

In this subsection, we first dissect the features of the presented model compared with existing models. Then, we address the question: to what extent are the insights presented here valid for other cases. This requires us to scrutinize assumptions and limitations of the model. Afterwards, we discuss modifications and extensions needed to implant the model for more general cases.

Taking a wider view, combining DFT calculations and microkinetic considerations in modelling electrocatalytic reactions are becoming increasingly popular.30, 31, 81 Some studies have further used mean-field EDL models to compute the local reaction condition for the ORR,29, 82, 83 and carbon dioxide reduction,33, 76. As for the OER, our model is, to our best knowledge, the first one combines DFT calculations, microkinetic considerations and an EDL model. Moreover, compared with similar models that were developed for other reactions,33, 76, 81, 82 this model has two unique features. Firstly, the EDL model considers nonlinear solvent polarization and ion size effect. Secondly, chemisorption-induced surface dipoles, significantly influencing the surface charging relation, are considered.

The assumptions and approximations of the model are four-fold. Firstly, the microkinetic model is developed based on the OER mechanism consisting of four OH--coupled electron transfer steps expressed in Eq.(14)-(17). Secondly, the phenomenological Butler-Volmer equation is used to describe electron transfer kinetics. Thirdly, the EDL is considered as a serial connection of a charged adlayer and a diffuse layer, and treated at the mean field level for the one-dimensional case. Fourthly, the electrolyte cations are restricted to be nonspecifically adsorbed. Fifthly, macroscopic mass transport effects are neglected.

On the first point, the used reaction mechanism is common to many OER catalysts, though the active sites, the binding energies, and the activation energies are case-specific. We have demonstrated in the preceding subsection that our results are robust against variations in these parameters. For OER catalysts with decoupled OH- and electron transfer steps, the microkinetic model can be modified without major difficulties.

On the second point, the Butler-Volmer equation can be upgraded to more advanced electron transfer theories, however, at the cost of introducing further microscopic parameters. For example, the Marcus-Hush-Chidsey theory requires the knowledge of interfacial solvent reorganization energies, and the Anderson-Newns models require the knowledge of electronic interactions between the catalyst and the reactant in solution. Therefore, the Butler-Volmer equation best suits our purpose. In addition, the Marcus-Hush-Chidsey theory can be reduced to the Butler-Volmer equation with a potential-varying transfer coefficient. Our sensitivity analysis shows that variations in the transfer coefficient do not alter the conclusion.

On the third point, the mean-field EDL model radically simplifies the atomistic picture of the EDL which can be obtained from DFT-based first-principles calculations.84, 85 Atomistic EDL models, such as in refs.86, 87,  can complement the mean-field EDL model by determining some of the model parameters, including the structure of the adlayer and the net charge numbers of adsorbed intermediates. We are aware that the ξi’s are difficult to obtain accurately and depend on the theoretical method used. Through sensitivity analysis, we have known that the cation effects are highly sensitive to ξi’s.

To address the fourth point, there is controversy around whether electrolyte cations are nonspecifically adsorbed,28, 88 specifically adsorbed,26, 89 or even intercalated into the electrode material.25 As for alkali metal cations at a NiOOH surface, Garcia et al. showed that the electrochemical active surface area (ECSA) is not influenced significantly by electrolyte cations,14 invalidating a previous postulate that the cation-dependent activity is related to the ECSA change due to cation intercalation.25 Crystal truncation rod analysis of Rao et al. indicates that Li+ and K+ are nonspecifically adsorbed and retain their solvation shells near the RuO2 surface.28 These are experimental evidences for the assumption of nonspecific adsorption in this model. For the case with specific adsorption of cations, its effect usually translates to a shift of binding energies.14, 26 For such cases, we recall that the findings of the present model are robust against variations in binding energies, prompting us to conclude that the present model would remain applicable for the case of cation specific adsorption.

On the fifth point, mass transport is one of the influencing factors of the polarization curves, and its importance is crucial in high overpotential region where the current density is rather high. The presented study is focused on the cation effect on the intrinsic catalytic activity in low overpotential region. Therefore, the mass transport effects are secondary in this work. A comprehensive physicochemical model for electrocatalytic reactions considering interfacial multistep kinetics, EDL effects, and macroscopic mass transport effects is developed recently.90

# Conclusion

We presented a physico-chemical model for understanding how electrolyte cations influence the OER via shaping the local reaction condition. The decreasing OER activity with larger cations was found to be correlated with cation overcrowding effects in the electric double layer. Specifically, the electrode surface is negatively charged due to the high pzc, resulting in cation accumulation and anion depletion near the electrode. Furthermore, cations with larger effective size occupy more volume, leading to smaller concentration of OH- and thus lower OER activity. The same reasoning has been employed to understand why the OER activity increases with solution pH on the RHE scale, and why the OER activity decreases in the presence of bivalent cations. The correction to the binding energies that depend on the local electric field plays a minor role in our case. A sensitivity analysis shows that the cation size effect is sensitive to properties related to the surface charging behavior, including the potential of zero charge and the net charge numbers of adsorbed intermediates. In addition, Monte-Carlo simulations demonstrated that the key findings reported are robust against uncertainties and variations in binding energies, activation barriers, and transfer coefficients. This work adds to the understanding of how the local reaction environment influences electrocatalytic activities.

# Author contribution

J.H. – Conceptualization, Investigation, Writing - Original Draft;

M.L. – DFT calculations, Writing - Review & Editing;

M.J. E. - DFT calculations, Writing - Review & Editing;

M.E. - Writing - Review & Editing, Supervision;

A.G. - Writing - Review & Editing, Supervision;

# Acknowledgement

J.H. acknowledges the financial support from the Alexander von Humboldt Foundation and National Natural Science Foundation of China under the grant number of 21802170. ME acknowledges the financial support from Forschungszentrum Jülich GmbH. Computer time provided by the state of Baden-Württemberg through bwHPC and the German Research Foundation (DFG) through grant no INST 40/575-1 FUGG (JUSTUS 2 cluster) are gratefully acknowledged. This work contributes to the research performed at CELEST (Center for Electrochemical Energy Storage Ulm-Karlsruhe).

# Supporting Information Available

Model fitting details; Results for an extreme case \with one step having a much larger activation energy; Zero-point vibrational corrections; Projected magnetic moments for oxygen and nickel atoms; Variations in experimental polarization curves on the same system measured by different laboratories

# Reference

1.	Nørskov, J. K.;  Bligaard, T.;  Logadottir, A.;  Kitchin, J. R.;  Chen, J. G.;  Pandelov, S.; Stimming, U., Trends in the Exchange Current for Hydrogen Evolution. Journal of The Electrochemical Society 2005, 152 (3), J23.

2.	Quaino, P.;  Juarez, F.;  Santos, E.; Schmickler, W., Volcano plots in hydrogen electrocatalysis – uses and abuses. Beilstein Journal of Nanotechnology 2014, 5, 846-854.

3.	Schmickler, W., On the Theory of Electrocatalysis. In Electrochemical Science for a Sustainable Society: A Tribute to John O’M Bockris, Uosaki, K., Ed. Springer International Publishing: Cham, 2017; pp 95-111.

4.	Exner, K. S., Why approximating electrocatalytic activity by a single free‐energy change is insufficient. Electrochimica Acta 2021, 375, 137975.

5.	Bandarenka, A. S.; Koper, M. T. M., Structural and electronic effects in heterogeneous electrocatalysis: Toward a rational design of electrocatalysts. Journal of Catalysis 2013, 308, 11-24.

6.	Nørskov, J. K.;  Bligaard, T.;  Rossmeisl, J.; Christensen, C. H., Towards the computational design of solid catalysts. Nature Chemistry 2009, 1 (1), 37-46.

7.	Strmcnik, D.;  Kodama, K.;  van der Vliet, D.;  Greeley, J.;  Stamenkovic, V. R.; Marković, N. M., The role of non-covalent interactions in electrocatalytic fuel-cell reactions on platinum. Nature Chemistry 2009, 1 (6), 466-472.

8.	Xue, S.;  Garlyyev, B.;  Watzele, S.;  Liang, Y.;  Fichtner, J.;  Pohl, M. D.; Bandarenka, A. S., Influence of Alkali Metal Cations on the Hydrogen Evolution Reaction Activity of Pt, Ir, Au, and Ag Electrodes in Alkaline Electrolytes. ChemElectroChem 2018, 5 (17), 2326-2329.

9.	Liu, E.;  Li, J.;  Jiao, L.;  Doan, H. T. T.;  Liu, Z.;  Zhao, Z.;  Huang, Y.;  Abraham, K. M.;  Mukerjee, S.; Jia, Q., Unifying the Hydrogen Evolution and Oxidation Reactions Kinetics in Base by Identifying the Catalytic Roles of Hydroxyl-Water-Cation Adducts. Journal of the American Chemical Society 2019, 141 (7), 3232-3239.

10.	Waegele, M. M.;  Gunathunge, C. M.;  Li, J.; Li, X., How cations affect the electric double layer and the rates and selectivity of electrocatalytic processes. The Journal of Chemical Physics 2019, 151 (16), 160902.

11.	Suntivich, J.;  Perry, E. E.;  Gasteiger, H. A.; Shao-Horn, Y., The Influence of the Cation on the Oxygen Reduction and Evolution Activities of Oxide Surfaces in Alkaline Electrolyte. Electrocatalysis 2013, 4 (1), 49-55.

12.	Tymoczko, J.;  Colic, V.;  Ganassin, A.;  Schuhmann, W.; Bandarenka, A. S., Influence of the alkali metal cations on the activity of Pt(111) towards model electrocatalytic reactions in acidic sulfuric media. Catalysis Today 2015, 244, 96-102.

13.	Zhu, S.;  Hu, X.;  Zhang, L.; Shao, M., Impacts of Perchloric Acid, Nafion, and Alkali Metal Ions on Oxygen Reduction Reaction Kinetics in Acidic and Alkaline Solutions. The Journal of Physical Chemistry C 2016, 120 (48), 27452-27461.

14.	Garcia, A. C.;  Touzalin, T.;  Nieuwland, C.;  Perini, N.; Koper, M. T. M., Enhancement of Oxygen Evolution Activity of Nickel Oxyhydroxide by Electrolyte Alkali Cations. Angewandte Chemie International Edition 2019, 58 (37), 12999-13003.

15.	Sakong, S.; Groß, A., The Importance of the Electrochemical Environment in the Electro-Oxidation of Methanol on Pt(111). ACS Catalysis 2016, 6 (8), 5575-5586.

16.	Resasco, J.;  Chen, L. D.;  Clark, E.;  Tsai, C.;  Hahn, C.;  Jaramillo, T. F.;  Chan, K.; Bell, A. T., Promoter Effects of Alkali Metal Cations on the Electrochemical Reduction of Carbon Dioxide. Journal of the American Chemical Society 2017, 139 (32), 11277-11287.

17.	Ludwig, T.;  Gauthier, J. A.;  Dickens, C. F.;  Brown, K. S.;  Ringe, S.;  Chan, K.; Nørskov, J. K., Atomistic Insight into Cation Effects on Binding Energies in Cu-Catalyzed Carbon Dioxide Reduction. The Journal of Physical Chemistry C 2020, 124 (45), 24765-24775.

18.	Erdey-Grúz, T.; Shafarik, I. In Soviet Electrochemistry, Proc. 4th Conf. Electrochemistry, Consultants Bureau New York: 1961; p 145.

19.	Kozawa, A., Effects of anions and cations on oxygen reduction and oxygen evolution reactions on platinum electrodes. Journal of Electroanalytical Chemistry (1959) 1964, 8 (1), 20-39.

20.	Armstrong, R. A.;  Briggs, G. W. D.; Moore, M. A., The effect of lithium in preventing iron poisoning in the nickel hydroxide electrode. Electrochimica Acta 1986, 31 (1), 25-27.

21.	Uñates, M. E.;  Folquer, M. E.;  Vilche, J. R.; Arvia, A. J., The Influence of Foreign Cations on the Electrochemical Behavior of the Nickel Hydroxide Electrode. Journal of The Electrochemical Society 1992, 139 (10), 2697-2704.

22.	Barnard, R.;  Randell, C. F.; Tye, F. L., Studies concerning charged nickel hydroxide electrodes IV. Reversible potentials in LiOH, NaOH, RbOH and CsOH. Journal of Applied Electrochemistry 1981, 11 (4), 517-523.

23.	Vishnu Kamath, P.; Ahmed, M. F., Cyclic voltammetric studies of nickel hydroxide and cobalt hydroxide thin films in alkali and alkaline earth metal hydroxides. Journal of Applied Electrochemistry 1993, 23 (3), 225-230.

24.	Michael, J. D.;  Demeter, E. L.;  Illes, S. M.;  Fan, Q.;  Boes, J. R.; Kitchin, J. R., Alkaline Electrolyte and Fe Impurity Effects on the Performance and Active-Phase Structure of NiOOH Thin Films for OER Catalysis Applications. The Journal of Physical Chemistry C 2015, 119 (21), 11475-11481.

25.	Zaffran, J.;  Stevens, M. B.;  Trang, C. D. M.;  Nagli, M.;  Shehadeh, M.;  Boettcher, S. W.; Caspary Toroker, M., Influence of Electrolyte Cations on Ni(Fe)OOH Catalyzed Oxygen Evolution Reaction. Chemistry of Materials 2017, 29 (11), 4761-4767.

26.	Yang, C.;  Fontaine, O.;  Tarascon, J.-M.; Grimaud, A., Chemical Recognition of Active Oxygen Species on the Surface of Oxygen Evolution Reaction Electrocatalysts. Angewandte Chemie International Edition 2017, 56 (30), 8652-8656.

27.	Perini, N.; Ticianelli, E. A., Oxygen evolution on gold: The effects of alkali-metal cations and iron impurities from alkaline electrolytes. Journal of Catalysis 2019, 378, 277-282.

28.	Rao, R. R.;  Huang, B.;  Katayama, Y.;  Hwang, J.;  Kawaguchi, T.;  Lunger, J. R.;  Peng, J.;  Zhang, Y.;  Morinaga, A.;  Zhou, H.;  You, H.; Shao-Horn, Y., pH- and Cation-Dependent Water Oxidation on Rutile RuO2(110). The Journal of Physical Chemistry C 2021, 125 (15), 8195-8207.

29.	Huang, J.;  Zhang, J.; Eikerling, M., Unifying theoretical framework for deciphering the oxygen reduction reaction on platinum. Physical Chemistry Chemical Physics 2018, 20 (17), 11776-11786.

30.	Dickens, C. F.;  Kirk, C.; Nørskov, J. K., Insights into the Electrochemical Oxygen Evolution Reaction with ab Initio Calculations and Microkinetic Modeling: Beyond the Limiting Potential Volcano. The Journal of Physical Chemistry C 2019, 123 (31), 18960-18977.

31.	Govind Rajan, A.; Carter, E. A., Microkinetic model for pH- and potential-dependent oxygen evolution during water splitting on Fe-doped β-NiOOH. Energy & Environmental Science 2020, 13 (12), 4962-4976.

32.	Sinha, V.;  Sun, D.;  Meijer, E. J.;  Vlugt, T. J. H.; Bieberle-Hütter, A., A multiscale modelling approach to elucidate the mechanism of the oxygen evolution reaction at the hematite–water interface. Faraday Discussions 2021,  (229), 89-107.

33.	Ringe, S.;  Morales-Guio, C. G.;  Chen, L. D.;  Fields, M.;  Jaramillo, T. F.;  Hahn, C.; Chan, K., Double layer charging driven carbon dioxide adsorption limits the rate of electrochemical carbon dioxide reduction on Gold. Nature Communications 2020, 11 (1), 33.

34.	Zhang, M.-K.;  Wei, Z.;  Chen, W.;  Xu, M.-L.;  Cai, J.; Chen, Y.-X., Bell shape vs volcano shape pH dependent kinetics of the electrochemical oxidation of formic acid and formate, intrinsic kinetics or local pH shift? Electrochimica Acta 2020, 363, 137160.

35.	Zhang, M.-K.;  Chen, W.;  Wei, Z.;  Xu, M.-L.;  He, Z.;  Cai, J.;  Chen, Y.-X.; Santos, E., Mechanistic Implication of the pH Effect and H/D Kinetic Isotope Effect on HCOOH/HCOO– Oxidation at Pt Electrodes: A Study by Computer Simulation. ACS Catalysis 2021, 6920-6930.

36.	Blöchl, P. E., Projector augmented-wave method. Physical Review B 1994, 50 (24), 17953-17979.

37.	Kresse, G.; Joubert, D., From ultrasoft pseudopotentials to the projector augmented-wave method. Physical Review B 1999, 59 (3), 1758-1775.

38.	Kresse, G.; Furthmüller, J., Efficient iterative schemes for ab initio total-energy calculations using a plane-wave basis set. Physical Review B 1996, 54 (16), 11169-11186.

39.	Kresse, G.; Furthmüller, J., Efficiency of ab-initio total energy calculations for metals and semiconductors using a plane-wave basis set. Computational Materials Science 1996, 6 (1), 15-50.

40.	Kresse, G.; Hafner, J., Ab initio molecular-dynamics simulation of the liquid-metal--amorphous-semiconductor transition in germanium. Physical Review B 1994, 49 (20), 14251-14269.

41.	Perdew, J. P.;  Burke, K.; Ernzerhof, M., Generalized Gradient Approximation Made Simple. Physical Review Letters 1996, 77 (18), 3865-3868.

42.	Dudarev, S. L.;  Botton, G. A.;  Savrasov, S. Y.;  Humphreys, C. J.; Sutton, A. P., Electron-energy-loss spectra and the structural stability of nickel oxide: An LSDA+U study. Physical Review B 1998, 57 (3), 1505-1509.

43.	Wang, L.;  Maxisch, T.; Ceder, G., Oxidation energies of transition metal oxides within the $\mathrm{GGA}+\mathrm{U}$ framework. Physical Review B 2006, 73 (19), 195107.

44.	Eslamibidgoli, M. J.;  Groß, A.; Eikerling, M., Surface configuration and wettability of nickel(oxy)hydroxides: a first-principles investigation. Physical Chemistry Chemical Physics 2017, 19 (34), 22659-22669.

45.	Li, Y.-F.; Selloni, A., Mechanism and Activity of Water Oxidation on Selected Surfaces of Pure and Fe-Doped NiOx. ACS Catalysis 2014, 4 (4), 1148-1153.

46.	Grimme, S.;  Ehrlich, S.; Goerigk, L., Effect of the damping function in dispersion corrected density functional theory. Journal of Computational Chemistry 2011, 32 (7), 1456-1465.

47.	Reuter, K.;  Frenkel, D.; Scheffler, M., The Steady State of Heterogeneous Catalysis, Studied by First-Principles Statistical Mechanics. Physical Review Letters 2004, 93 (11), 116105.

48.	Reuter, K.; Scheffler, M., First-principles kinetic Monte Carlo simulations for heterogeneous catalysis: Application to the CO oxidation at $\mathrm{Ru}{\mathrm{O}}_{2}(110)$. Physical Review B 2006, 73 (4), 045433.

49.	Nørskov, J. K.;  Rossmeisl, J.;  Logadottir, A.;  Lindqvist, L.;  Kitchin, J. R.;  Bligaard, T.; Jónsson, H., Origin of the Overpotential for Oxygen Reduction at a Fuel-Cell Cathode. The Journal of Physical Chemistry B 2004, 108 (46), 17886-17892.

50.	Groß, A., Grand-canonical approaches to understand structures and processes at electrochemical interfaces from an atomistic perspective. Current Opinion in Electrochemistry 2021, 27, 100684.

51.	Diaz-Morales, O.;  Ferrus-Suspedra, D.; Koper, M. T. M., The importance of nickel oxyhydroxide deprotonation on its activity towards electrochemical water oxidation. Chemical Science 2016, 7 (4), 2639-2645.

52.	Lee, S.;  Banjac, K.;  Lingenfelder, M.; Hu, X., Oxygen Isotope Labeling Experiments Reveal Different Reaction Sites for the Oxygen Evolution Reaction on Nickel and Nickel Iron Oxides. Angewandte Chemie International Edition 2019, 58 (30), 10295-10299.

53.	Linstrom, P. J.; Mallard, W. G., The NIST Chemistry WebBook: A chemical data resource on the internet. Journal of Chemical Engineering Data 2001, 46 (5), 1059-1063.

54.	Govind Rajan, A.;  Martirez, J. M. P.; Carter, E. A., Facet-Independent Oxygen Evolution Activity of Pure β-NiOOH: Different Chemistries Leading to Similar Overpotentials. Journal of the American Chemical Society 2020, 142 (7), 3600-3612.

55.	Rossmeisl, J.;  Qu, Z. W.;  Zhu, H.;  Kroes, G. J.; Nørskov, J. K., Electrolysis of water on oxide surfaces. Journal of Electroanalytical Chemistry 2007, 607 (1), 83-89.

56.	Man, I. C.;  Su, H.-Y.;  Calle-Vallejo, F.;  Hansen, H. A.;  Martínez, J. I.;  Inoglu, N. G.;  Kitchin, J.;  Jaramillo, T. F.;  Nørskov, J. K.; Rossmeisl, J., Universality in Oxygen Evolution Electrocatalysis on Oxide Surfaces. ChemCatChem 2011, 3 (7), 1159-1165.

57.	Divanis, S.;  Kutlusoy, T.;  Ingmer Boye, I. M.;  Man, I. C.; Rossmeisl, J., Oxygen evolution reaction: a perspective on a decade of atomic scale simulations. Chemical Science 2020, 11 (11), 2943-2950.

58.	Bajpai, A.;  Frey, K.; Schneider, W. F., Comparison of Coverage-Dependent Binding Energy Models for Mean-Field Microkinetic Rate Predictions. Langmuir 2020, 36 (1), 465-474.

59.	Chen, L. D.;  Urushihara, M.;  Chan, K.; Nørskov, J. K., Electric Field Effects in Electrochemical CO2 Reduction. ACS Catalysis 2016, 6 (10), 7133-7139.

60.	Sandberg, R. B.;  Montoya, J. H.;  Chan, K.; Nørskov, J. K., CO-CO coupling on Cu facets: Coverage, strain and field effects. Surface Science 2016, 654, 56-62.

61.	Huang, J., Hybrid Density-Potential Functional Theory of Electric Double Layers. Electrochimica Acta 2021, 138720.

62.	Huang, J.;  Chen, S.; Eikerling, M., Grand-Canonical Model of Electrochemical Double Layers from a Hybrid Density–Potential Functional. Journal of Chemical Theory and Computation 2021, 17 (4), 2417-2430.

63.	Abrashkin, A.;  Andelman, D.; Orland, H., Dipolar Poisson-Boltzmann Equation: Ions and Dipoles Close to Charge Interfaces. Physical Review Letters 2007, 99 (7), 077801.

64.	Gongadze, E.; Iglič, A., Decrease of permittivity of an electrolyte solution near a charged surface due to saturation and excluded volume effects. Bioelectrochemistry 2012, 87, 199-203.

65.	Zhang, Y.; Huang, J., Treatment of Ion-Size Asymmetry in Lattice-Gas Models for Electrical Double Layer. J. Phys. Chem. C 2018, 122 (50), 28652-28664.

66.	Huang, J.;  Malek, A.;  Zhang, J.; Eikerling, M. H., Non-monotonic Surface Charging Behavior of Platinum: A Paradigm Change. J. Phys. Chem. C 2016, 120 (25), 13587-13595.

67.	Huang, J.;  Zhou, T.;  Zhang, J.; Eikerling, M., Double layer of platinum electrodes: Non-monotonic surface charging phenomena and negative double layer capacitance. J. Chem. Phys. 2018, 148 (4), 044704.

68.	Israelachvili, J. N., Intermolecular and surface forces. Academic press: 2015.

69.	Wellendorff, J.;  Silbaugh, T. L.;  Garcia-Pintos, D.;  Nørskov, J. K.;  Bligaard, T.;  Studt, F.; Campbell, C. T., A benchmark database for adsorption bond energies to transition metal surfaces and comparison to selected DFT functionals. Surface Science 2015, 640, 36-44.

70.	Huang, J., Mixed quantum-classical treatment of electron transfer at electrocatalytic interfaces: Theoretical framework and conceptual analysis. The Journal of Chemical Physics 2020, 153 (16), 164707.

71.	Guidelli, R.;  Compton, R. G.;  Feliu, J. M.;  Gileadi, E.;  Lipkowski, J.;  Schmickler, W.; Trasatti, S., Defining the transfer coefficient in electrochemistry: An assessment (IUPAC Technical Report). Pure and Applied Chemistry 2014, 86 (2), 245-258.

72.	Henkelman, G.;  Uberuaga, B. P.; Jónsson, H., A climbing image nudged elastic band method for finding saddle points and minimum energy paths. The Journal of Chemical Physics 2000, 113 (22), 9901-9904.

73.	Kosmulski, M., Isoelectric points and points of zero charge of metal (hydr)oxides: 50years after Parks' review. Advances in Colloid and Interface Science 2016, 238, 1-61.

74.	Nakayama, Y.; Andelman, D., Differential capacitance of the electric double layer: The interplay between ion finite size and dielectric decrement. The Journal of Chemical Physics 2015, 142 (4), 044706.

75.	Borukhov, I.;  Andelman, D.; Orland, H., Steric Effects in Electrolytes: A Modified Poisson-Boltzmann Equation. Phys. Rev. Lett. 1997, 79 (3), 435-438.

76.	Ringe, S.;  Clark, E. L.;  Resasco, J.;  Walton, A.;  Seger, B.;  Bell, A. T.; Chan, K., Understanding cation effects in electrochemical CO2 reduction. Energy & Environmental Science 2019, 12 (10), 3001-3014.

77.	Koper, M. T. M., Theory of multiple proton–electron transfer reactions and its implications for electrocatalysis. Chemical Science 2013, 4 (7), 2710-2723.

78.	Giordano, L.;  Han, B.;  Risch, M.;  Hong, W. T.;  Rao, R. R.;  Stoerzinger, K. A.; Shao-Horn, Y., pH dependence of OER activity of oxides: Current and future perspectives. Catalysis Today 2016, 262, 2-10.

79.	Grimaud, A.;  Diaz-Morales, O.;  Han, B.;  Hong, W. T.;  Lee, Y.-L.;  Giordano, L.;  Stoerzinger, K. A.;  Koper, M. T. M.; Shao-Horn, Y., Activating lattice oxygen redox reactions in metal oxides to catalyse oxygen evolution. Nature Chemistry 2017, 9 (5), 457-465.

80.	Duan, Y.;  Dubouis, N.;  Huang, J.;  Dalla Corte, D. A.;  Pimenta, V.;  Xu, Z. J.; Grimaud, A., Revealing the Impact of Electrolyte Composition for Co-Based Water Oxidation Catalysts by the Study of Reaction Kinetics Parameters. ACS Catalysis 2020, 10 (7), 4160-4170.

81.	Huang, J.; Eikerling, M., Modeling the oxygen reduction reaction at platinum-based catalysts: A brief review of recent developments. Current Opinion in Electrochemistry 2019, 13, 157-165.

82.	Zhou, D.;  Wei, J.;  He, Z.-D.;  Xu, M.-L.;  Chen, Y.-X.; Huang, J., Combining Single Crystal Experiments and Microkinetic Modeling in Disentangling Thermodynamic, Kinetic, and Double-Layer Factors Influencing Oxygen Reduction. The Journal of Physical Chemistry C 2020, 124 (25), 13672-13678.

83.	Kelly, S. R.;  Kirk, C.;  Chan, K.; Nørskov, J. K., Electric Field Effects in Oxygen Reduction Kinetics: Rationalizing pH Dependence at the Pt(111), Au(111), and Au(100) Electrodes. The Journal of Physical Chemistry C 2020, 124 (27), 14581-14591.

84.	Abidi, N.;  Lim, K. R. G.;  Seh, Z. W.; Steinmann, S. N., Atomistic modeling of electrocatalysis: Are we there yet? WIREs Computational Molecular Science 2020, n/a (n/a), e1499.

85.	Magnussen, O. M.; Groß, A., Toward an Atomic-Scale Understanding of Electrochemical Interface Structure and Dynamics. J. Am. Chem. Soc. 2019, 141 (12), 4777-4790.

86.	Sakong, S.; Groß, A., The electric double layer at metal-water interfaces revisited based on a charge polarization scheme. J. Chem. Phys. 2018, 149 (8), 084705.

87.	Sakong, S.; Groß, A., Water structures on a Pt(111) electrode from ab initio molecular dynamic simulations for a variety of electrochemical conditions. Phys. Chem. Chem. Phys. 2020, 22 (19), 10431-10437.

88.	Strmcnik, D.;  van der Vliet, D. F.;  Chang, K. C.;  Komanicky, V.;  Kodama, K.;  You, H.;  Stamenkovic, V. R.; Marković, N. M., Effects of Li+, K+, and Ba2+ Cations on the ORR at Model and High Surface Area Pt and Au Surfaces in Alkaline Solutions. The Journal of Physical Chemistry Letters 2011, 2 (21), 2733-2736.

89.	Mills, J. N.;  McCrum, I. T.; Janik, M. J., Alkali cation specific adsorption onto fcc(111) transition metal electrodes. Physical Chemistry Chemical Physics 2014, 16 (27), 13699-13707.

90.	Zhang, L.;  Cai, J.;  Chen, Y.; Huang, J., Modelling electrocatalytic reactions with a concerted treatment of multistep electron transfer kinetics and local reaction conditions. Journal of Physics: Condensed Matter 2021,  (Special issue "Physical of Electrocatalysis"), Revised.